taskYIELD()

- Trước khi gọi → task đang ở Running.

- Sau khi gọi:

  - Scheduler sẽ đưa task gọi taskYIELD() về Ready (vẫn ở mức ưu tiên cũ).

  - Nếu có task khác cùng/ưu tiên cao hơn trong Ready → context switch xảy ra, task gọi taskYIELD() sẽ rời khỏi Running và vào Ready.

  - Nếu không có task nào khác có thể chạy → scheduler lại chọn chính task đó → nó tiếp tục ở Running (coi như không đổi).

context switch (chuyển ngữ cảnh): quá trình dừng một task đang chạy và chuyển CPU sang cho một task khác

+ Lưu context task hiện tại vào stack riêng của nó.

+ Cập nhật Task Control Block (TCB) để biết task này đang ở trạng thái gì, stack pointer hiện tại ở đâu.

+ Chọn task khác trong Ready List theo thuật toán scheduler (priority-based, round robin cùng priority).

+ Nạp lại context của task mới từ stack → CPU tiếp tục chạy task đó.

FreeRTOS event groups

Event Bits (Event Flags)

Event bits được sử dụng để cho biết an event đã xảy ra hoặc không. Các bit sự kiện thường được gọi là cờ sự kiện. Event bits are often referred to as event flags.

Event Bits (Event Flags)

An event group is a set of event bits. Event bits riêng lẻ trong an event group được tham chiếu bằng một số bit.

Event Group and Event Bits Data Types

Event groups được tham chiếu bởi các biến thuộc loại EventGroupHandle_t.

Số bit (hoặc cờ) được lưu trữ trong một nhóm sự kiện là 8 nếu configUSE_16_BIT_TICKS được đặt thành 1 hoặc 24 đặt thành 0. Sự phụ thuộc vào configUSE_16_BIT_TICKS là kết quả của kiểu dữ liệu được sử dụng cho lưu trữ cục bộ luồng trong việc thực hiện các tác vụ nội bộ.

Tất cả các bit sự kiện trong một nhóm sự kiện được lưu trữ trong một biến không dấu duy nhất của EventBits_t. Bit sự kiện 0 được lưu trữ ở vị trí bit 0, bit sự kiện 1 là được lưu trữ ở vị trí bit 1, v.v.

Hình ảnh dưới đây đại diện cho một nhóm sự kiện 24-bit sử dụng ba bit để Giữ ba sự kiện ví dụ đã được mô tả. Trong hình ảnh chỉ có sự kiện bit 2 được thiết lập.

Event Group RTOS API Functions

Event group API functions  set one or more event bits within an event group, clear one or more event bits within an event group, and pend (enter the Blocked state để không tiêu tốn bất kỳ thời gian xử lý nào) to wait for a set of one or more event bits to become set within an event group.

Event groups can also be used to synchronise tasks (đồng bộ hóa task), creating what is often referred to as a task 'rendezvous' 'điểm hẹn'. A task synchronisation point is a place in application code at which a task will wait in the Blocked state (không tiêu tốn bất kỳ thời gian CPU nào) until all the other tasks taking part in the synchronisation also reached their synchronisation point.

xEventGroupSetBits() = đặt một cờ trong Event Group để báo hiệu cho các task khác (đang chờ bằng xEventGroupWaitBits()) rằng một sự kiện đã xảy ra.

Task nào đang chờ bit WIFI_CONNECTED_BIT thì bây giờ có thể tiếp tục

SCAN_WIFI

- Khởi tạo Wi-Fi ở chế độ STA (Station).

- Thực hiện lệnh quét (scan) các Access Point (AP) xung quanh.

- In ra danh sách SSID, RSSI, kiểu mã hóa (Auth mode, Cipher type), kênh hoạt động.

static uint8_t channel_list[CHANNEL_LIST_SIZE] = {1, 6, 11};

 ESP32 có thể scan tất cả kênh 1–13 (thậm chí 14 ở Nhật).

 Nhưng 1, 6, 11 là 3 kênh chính, không chồng lấn, nên thường được dùng nhất.

 Dùng chúng giúp tránh tình trạng WiFi chồng sóng và mất ổn định.

/* Scan Example

This example code is in the Public Domain (or CC0 licensed, at your option.)

Unless required by applicable law or agreed to in writing, this

software is distributed on an "AS IS" BASIS, WITHOUT WARRANTIES OR

CONDITIONS OF ANY KIND, either express or implied.

*/

/*

This example shows how to scan for available set of APs.

*/

#include <string.h>

#include "freertos/FreeRTOS.h"

#include "freertos/event_groups.h"

#include "esp_wifi.h"

#include "esp_log.h"

#include "esp_event.h"

#include "nvs_flash.h"

#include "regex.h"

#include "lwip/err.h"

#include "lwip/sys.h"

#define DEFAULT_SCAN_LIST_SIZE 10

#ifdef CONFIG_EXAMPLE_USE_SCAN_CHANNEL_BITMAP

#define USE_CHANNEL_BITMAP 1

#define CHANNEL_LIST_SIZE 3

static uint8_t channel_list[CHANNEL_LIST_SIZE] = {1, 6, 11};

#endif /*CONFIG_EXAMPLE_USE_SCAN_CHANNEL_BITMAP*/

#define BLINK_GPIO GPIO_NUM_2

static const char *TAG = "scan";

static void print_auth_mode(int authmode)

{

switch (authmode) {

case WIFI_AUTH_OPEN:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_OPEN");

break;

case WIFI_AUTH_OWE:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_OWE");

break;

case WIFI_AUTH_WEP:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WEP");

break;

case WIFI_AUTH_WPA_PSK:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA_PSK");

break;

case WIFI_AUTH_WPA2_PSK:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA2_PSK");

break;

case WIFI_AUTH_WPA_WPA2_PSK:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA_WPA2_PSK");

break;

case WIFI_AUTH_ENTERPRISE:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_ENTERPRISE");

break;

case WIFI_AUTH_WPA3_PSK:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA3_PSK");

break;

case WIFI_AUTH_WPA2_WPA3_PSK:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA2_WPA3_PSK");

break;

case WIFI_AUTH_WPA3_ENTERPRISE:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA3_ENTERPRISE");

break;

case WIFI_AUTH_WPA2_WPA3_ENTERPRISE:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA2_WPA3_ENTERPRISE");

break;

case WIFI_AUTH_WPA3_ENT_192:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_WPA3_ENT_192");

break;

default:

ESP_LOGI(TAG, "Authmode \tWIFI_AUTH_UNKNOWN");

break;

}

}

static void print_cipher_type(int pairwise_cipher, int group_cipher)

{

switch (pairwise_cipher) {

case WIFI_CIPHER_TYPE_NONE:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_NONE");

break;

case WIFI_CIPHER_TYPE_WEP40:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_WEP40");

break;

case WIFI_CIPHER_TYPE_WEP104:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_WEP104");

break;

case WIFI_CIPHER_TYPE_TKIP:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_TKIP");

break;

case WIFI_CIPHER_TYPE_CCMP:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_CCMP");

break;

case WIFI_CIPHER_TYPE_TKIP_CCMP:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_TKIP_CCMP");

break;

case WIFI_CIPHER_TYPE_AES_CMAC128:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_AES_CMAC128");

break;

case WIFI_CIPHER_TYPE_SMS4:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_SMS4");

break;

case WIFI_CIPHER_TYPE_GCMP:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_GCMP");

break;

case WIFI_CIPHER_TYPE_GCMP256:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_GCMP256");

break;

default:

ESP_LOGI(TAG, "Pairwise Cipher \tWIFI_CIPHER_TYPE_UNKNOWN");

break;

}

switch (group_cipher) {

case WIFI_CIPHER_TYPE_NONE:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_NONE");

break;

case WIFI_CIPHER_TYPE_WEP40:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_WEP40");

break;

case WIFI_CIPHER_TYPE_WEP104:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_WEP104");

break;

case WIFI_CIPHER_TYPE_TKIP:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_TKIP");

break;

case WIFI_CIPHER_TYPE_CCMP:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_CCMP");

break;

case WIFI_CIPHER_TYPE_TKIP_CCMP:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_TKIP_CCMP");

break;

case WIFI_CIPHER_TYPE_SMS4:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_SMS4");

break;

case WIFI_CIPHER_TYPE_GCMP:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_GCMP");

break;

case WIFI_CIPHER_TYPE_GCMP256:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_GCMP256");

break;

default:

ESP_LOGI(TAG, "Group Cipher \tWIFI_CIPHER_TYPE_UNKNOWN");

break;

}

}

#ifdef USE_CHANNEL_BITMAP

static void array_2_channel_bitmap(const uint8_t channel_list[], const uint8_t channel_list_size, wifi_scan_config_t *scan_config) {

for(uint8_t i = 0; i < channel_list_size; i++) {

uint8_t channel = channel_list[i];

scan_config->channel_bitmap.ghz_2_channels |= (1 << channel);

}

}

#endif /*USE_CHANNEL_BITMAP*/

/* Initialize Wi-Fi as sta and set scan method */

static void wifi_scan(void)

{

ESP_ERROR_CHECK(esp_netif_init());

ESP_ERROR_CHECK(esp_event_loop_create_default());

esp_netif_t *sta_netif = esp_netif_create_default_wifi_sta();

assert(sta_netif);

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

ESP_ERROR_CHECK(esp_wifi_init(&cfg));

uint16_t number = DEFAULT_SCAN_LIST_SIZE;

wifi_ap_record_t ap_info[DEFAULT_SCAN_LIST_SIZE];

uint16_t ap_count = 0;

memset(ap_info, 0, sizeof(ap_info));

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_start());

#ifdef USE_CHANNEL_BITMAP

wifi_scan_config_t *scan_config = (wifi_scan_config_t *)calloc(1,sizeof(wifi_scan_config_t));

if (!scan_config) {

ESP_LOGE(TAG, "Memory Allocation for scan config failed!");

return;

}

array_2_channel_bitmap(channel_list, CHANNEL_LIST_SIZE, scan_config);

esp_wifi_scan_start(scan_config, true);

free(scan_config);

#else

esp_wifi_scan_start(NULL, true);

#endif /*USE_CHANNEL_BITMAP*/

ESP_LOGI(TAG, "Max AP number ap_info can hold = %u", number);

ESP_ERROR_CHECK(esp_wifi_scan_get_ap_num(&ap_count));

ESP_ERROR_CHECK(esp_wifi_scan_get_ap_records(&number, ap_info));

ESP_LOGI(TAG, "Total APs scanned = %u, actual AP number ap_info holds = %u", ap_count, number);

for (int i = 0; i < number; i++) {

ESP_LOGI(TAG, "SSID \t\t%s", ap_info[i].ssid);

ESP_LOGI(TAG, "RSSI \t\t%d", ap_info[i].rssi);

print_auth_mode(ap_info[i].authmode);

if (ap_info[i].authmode != WIFI_AUTH_WEP) {

print_cipher_type(ap_info[i].pairwise_cipher, ap_info[i].group_cipher);

}

ESP_LOGI(TAG, "Channel \t\t%d", ap_info[i].primary);

}

}

void get_input(const char *prompt, char *buffer, int max_len) {

char ESP_WIFI_SSID[32];

char ESP_WIFI_PASS[64];

printf("%s", prompt);              // In ra Serial Monitor

fflush(stdout);                    // Đảm bảo in ngay

fgets(buffer, max_len, stdin);     // Đọc từ UART0 (Serial)

// Xóa ký tự xuống dòng '\n' do người dùng gõ Enter

buffer[strcspn(buffer, "\n")] = 0;

}

static EventGroupHandle_t s_wifi_event_group;

#define WIFI_CONNECTED_BIT BIT0

#define WIFI_FAIL_BIT BIT1

#define ESP_MAXIMUM_RETRY 10

static const char *TAG_WIFI = "wifi station";

static int s_retry_num = 0;

static void event_handler(void* arg, esp_event_base_t event_base, int32_t event_id, void* event_data)

{

if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_START) {

esp_wifi_connect();

}

else if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_DISCONNECTED) {

if (s_retry_num < ESP_MAXIMUM_RETRY) {

esp_wifi_connect();

s_retry_num++;

ESP_LOGI(TAG_WIFI, "retry to connect to the AP");

} else {

xEventGroupSetBits(s_wifi_event_group, WIFI_FAIL_BIT);

}

ESP_LOGI(TAG_WIFI, "connect to the AP fail");

} else if (event_base == IP_EVENT && event_id == IP_EVENT_STA_GOT_IP) {

ip_event_got_ip_t* event = (ip_event_got_ip_t*) event_data;

ESP_LOGI(TAG_WIFI, "got ip:" IPSTR, IP2STR(&event->ip_info.ip));

s_retry_num = 0;

xEventGroupSetBits(s_wifi_event_group, WIFI_CONNECTED_BIT);

}

}

TaskHandle_t LEDTaskHandle = NULL;

void LED_Task(void *arg)

{

esp_rom_gpio_pad_select_gpio(BLINK_GPIO);

gpio_set_direction(BLINK_GPIO, GPIO_MODE_OUTPUT);

while(1) {

gpio_set_level(BLINK_GPIO, 1);

taskYIELD();

}

}

void wifi_init_sta(void)

{

s_wifi_event_group = xEventGroupCreate();

ESP_ERROR_CHECK(esp_netif_init());

ESP_ERROR_CHECK(esp_event_loop_create_default());

esp_netif_create_default_wifi_sta();

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

ESP_ERROR_CHECK(esp_wifi_init(&cfg));

esp_event_handler_instance_t instance_any_id;

esp_event_handler_instance_t instance_got_ip;

ESP_ERROR_CHECK(esp_event_handler_instance_register(WIFI_EVENT, ESP_EVENT_ANY_ID, &event_handler, NULL, &instance_any_id));

ESP_ERROR_CHECK(esp_event_handler_instance_register(IP_EVENT, IP_EVENT_STA_GOT_IP, &event_handler, NULL, &instance_got_ip));

wifi_config_t wifi_config = {

.sta = {

.ssid = ESP_WIFI_SSID,

.password = ESP_WIFI_PASS,

.threshold.authmode = WIFI_AUTH_WPA2_PSK,

},

};

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_set_config(WIFI_IF_STA, &wifi_config));

ESP_ERROR_CHECK(esp_wifi_start());

ESP_LOGI(TAG_WIFI, "wifi init start finished.");

EventBits_t bits = xEventGroupWaitBits(s_wifi_event_group, WIFI_CONNECTED_BIT | WIFI_FAIL_BIT, pdFALSE, pdFALSE, portMAX_DELAY);

if (bits & WIFI_CONNECTED_BIT) {

ESP_LOGI(TAG_WIFI, "connected to ap SSID:%s password:%s", ESP_WIFI_SSID, ESP_WIFI_PASS);

xTaskCreate(LED_Task, "LED", 4096, NULL, 10, &LEDTaskHandle);

} else if (bits & WIFI_FAIL_BIT) {

ESP_LOGI(TAG_WIFI, "Fail to connect to SSID:%s", ESP_WIFI_SSID);

} else {

ESP_LOGI(TAG_WIFI, "UNEXPECTED EVENT");

}

ESP_ERROR_CHECK(esp_event_handler_instance_unregister(IP_EVENT, IP_EVENT_STA_GOT_IP, instance_got_ip));

ESP_ERROR_CHECK(esp_event_handler_instance_unregister(WIFI_EVENT, ESP_EVENT_ANY_ID, instance_any_id));

vEventGroupDelete(s_wifi_event_group);

}

wifi_config_t wifi_config = {0};

strncpy((char *)wifi_config.sta.ssid, ssid, sizeof(wifi_config.sta.ssid));

strncpy((char *)wifi_config.sta.password, password, sizeof(wifi_config.sta.password));

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_set_config(WIFI_IF_STA, &wifi_config));

ESP_ERROR_CHECK(esp_wifi_connect());

void app_main(void)

{

// Initialize NVS

esp_err_t ret = nvs_flash_init();

if (ret == ESP_ERR_NVS_NO_FREE_PAGES || ret == ESP_ERR_NVS_NEW_VERSION_FOUND) {

ESP_ERROR_CHECK(nvs_flash_erase());

ret = nvs_flash_init();

}

ESP_ERROR_CHECK( ret );

wifi_scan();

get_input("Nhap SSID: ", ESP_WIFI_SSID, sizeof(ESP_WIFI_SSID));

get_input("Nhap Password: ", ESP_WIFI_PASS, sizeof(ESP_WIFI_PASS));

printf("SSID ban nhap: %s\n", ESP_WIFI_SSID);

printf("Password ban nhap: %s\n", ESP_WIFI_PASS);

wifi_init_stawifi_init_sta();

}

#include <stdio.h>

#include <string.h>

#include "freertos/FreeRTOS.h"

#include "freertos/task.h"

#include "freertos/event_groups.h"

#include "esp_wifi.h"

#include "esp_event.h"

#include "esp_log.h"

#include "nvs_flash.h"

#include "driver/gpio.h"

#define LED_GPIO 2

#define WIFI_CONNECTED_BIT BIT0

#define WIFI_FAIL_BIT      BIT1

static EventGroupHandle_t s_wifi_event_group;

static const char *TAG = "WiFi_Serial";

static void event_handler(void* arg, esp_event_base_t event_base,

int32_t event_id, void* event_data) {

if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_START) {

esp_wifi_connect();

} else if (event_base == WIFI_EVENT && event_id == WIFI_EVENT_STA_DISCONNECTED) {

ESP_LOGI(TAG, "Disconnected. Trying to reconnect...");

esp_wifi_connect();

xEventGroupClearBits(s_wifi_event_group, WIFI_CONNECTED_BIT);

} else if (event_base == IP_EVENT && event_id == IP_EVENT_STA_GOT_IP) {

ip_event_got_ip_t* event = (ip_event_got_ip_t*) event_data;

ESP_LOGI(TAG, "Got IP: " IPSTR, IP2STR(&event->ip_info.ip));

xEventGroupSetBits(s_wifi_event_group, WIFI_CONNECTED_BIT);

}

}

static void wifi_scan_and_print() {

// Cấu hình scan

wifi_scan_config_t scan_config = {

.ssid = 0,

.bssid = 0,

.channel = 0,

.show_hidden = true

};

printf("\nStarting WiFi Scan...\n");

ESP_ERROR_CHECK(esp_wifi_scan_start(&scan_config, true));  // true = block cho đến khi scan xong

// Lấy kết quả scan

uint16_t ap_count = 0;

ESP_ERROR_CHECK(esp_wifi_scan_get_ap_num(&ap_count));

wifi_ap_record_t ap_info[20];

uint16_t number = (ap_count < 20) ? ap_count : 20;

ESP_ERROR_CHECK(esp_wifi_scan_get_ap_records(&number, ap_info));

printf("Scan done, APs found: %d\n", ap_count);

for (int i = 0; i < number; i++) {

printf("%d: SSID: %s, RSSI: %d, Channel: %d\n",

i + 1,

(char *)ap_info[i].ssid,

ap_info[i].rssi,

ap_info[i].primary);

}

}

static void wifi_init_sta(const char *ssid, const char *pass) {

s_wifi_event_group = xEventGroupCreate();

esp_netif_create_default_wifi_sta();

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

ESP_ERROR_CHECK(esp_wifi_init(&cfg));

esp_event_handler_instance_t instance_any_id;

esp_event_handler_instance_t instance_got_ip;

ESP_ERROR_CHECK(esp_event_handler_instance_register(WIFI_EVENT,

ESP_EVENT_ANY_ID,

&event_handler,

NULL,

&instance_any_id));

ESP_ERROR_CHECK(esp_event_handler_instance_register(IP_EVENT,

IP_EVENT_STA_GOT_IP,

&event_handler,

NULL,

&instance_got_ip));

wifi_config_t wifi_config = { 0 };

strcpy((char *)wifi_config.sta.ssid, ssid);

strcpy((char *)wifi_config.sta.password, pass);

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_set_config(WIFI_IF_STA, &wifi_config));

ESP_ERROR_CHECK(esp_wifi_start());

EventBits_t bits = xEventGroupWaitBits(s_wifi_event_group,

WIFI_CONNECTED_BIT | WIFI_FAIL_BIT,

pdFALSE,

pdFALSE,

portMAX_DELAY);

if (bits & WIFI_CONNECTED_BIT) {

ESP_LOGI(TAG, "Connected to SSID:%s", ssid);

gpio_set_level(LED_GPIO, 1);  // Bật LED khi kết nối thành công

} else {

ESP_LOGI(TAG, "Failed to connect to SSID:%s", ssid);

gpio_set_level(LED_GPIO, 0);

}

}

void app_main(void) {

// Init NVS

esp_err_t ret = nvs_flash_init();

if (ret == ESP_ERR_NVS_NO_FREE_PAGES || ret == ESP_ERR_NVS_NEW_VERSION_FOUND) {

ESP_ERROR_CHECK(nvs_flash_erase());

ret = nvs_flash_init();

}

ESP_ERROR_CHECK(ret);

// Init LED

gpio_reset_pin(LED_GPIO);

gpio_set_direction(LED_GPIO, GPIO_MODE_OUTPUT);

gpio_set_level(LED_GPIO, 0);

ESP_ERROR_CHECK(esp_netif_init());

ESP_ERROR_CHECK(esp_event_loop_create_default());

// WiFi init

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();

ESP_ERROR_CHECK(esp_wifi_init(&cfg));

ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_STA));

ESP_ERROR_CHECK(esp_wifi_start());

// Scan WiFi

wifi_scan_and_print();

// Nhập SSID và Password từ Serial

char ssid[32];

char pass[64];

printf("\nEnter WiFi SSID: ");

scanf("%31s", ssid);

printf("Enter WiFi Password: ");

scanf("%63s", pass);

printf("\nYou entered:\nSSID: %s\nPassword: %s\n", ssid, pass);

// Connect WiFi

wifi_init_sta(ssid, pass);

}